# ==============================================================================================

# HEX/DEC Converter Application for RISC-V Single-Cycle Processor

# ==============================================================================================

# Description:

#   - Reads a 12-bit value from switches SW[11:0]

#   - Displays the value in both DECIMAL (HEX0-3) and HEXADECIMAL (HEX4-7) on 7-segment displays

#   - SW[12]: Master enable (1=display, 0=clear all)

#   - SW[13]: DEC display enable (1=show, 0=hide HEX0-3)

#   - SW[14]: HEX display enable (1=show, 0=hide HEX4-7)

#   - Red LEDs always show SW[14:0] regardless of enable state

#

# Memory Map:

#   SW_BASE   = 0x10010000  (Switch input register)

#   LEDR_BASE = 0x10000000  (Red LED output register)

#   HEX_LO    = 0x10002000  (7-segment displays HEX0-3)

#   HEX_HI    = 0x10003000  (7-segment displays HEX4-7)

# ==============================================================================================

_start:

# ===== Initialize Base Addresses =====

lui     s11, 0x10010       # s11 = 0x10010000 (Switch base address)

lui     t4, 0x10002        # t4  = 0x10002000 (HEX_LO: 7-seg displays 0-3)

lui     t5, 0x10003        # t5  = 0x10003000 (HEX_HI: 7-seg displays 4-7)

lui     s9, 0x10000        # s9  = 0x10000000 (Red LED base address)

main_loop:

# ===== Read Switch Values =====

lw      s8, 0(s11)         # s8 = 32-bit switch value

# ===== Update Red LEDs with SW[14:0] (Always Active) =====

# Extract lower 15 bits and display on red LEDs

addi    s10, s8, 0         # Copy switch value to s10

slli    s10, s10, 17       # Shift left 17 bits to clear upper bits

srli    s10, s10, 17       # Shift right 17 bits to isolate SW[14:0]

sw      s10, 0(s9)         # Write to LEDR register

# ===== Check Master Enable Bit (SW[12]) =====

srli    t6, s8, 12         # Shift right to get SW[12]

andi    t6, t6, 1          # Mask to get only bit 12: t6 = SW[12] (0 or 1)

beq     t6, x0, disabled   # If enable == 0, jump to disabled state

# ===== Extract 12-bit Input Value (SW[11:0]) =====

addi    t1, s8, 0          # Copy switch value

slli    t1, t1, 20         # Shift left 20 bits to clear upper 20 bits

srli    t1, t1, 20         # Shift right 20 bits to get SW[11:0]

# Note: Using shift instead of 'andi t1, s8, 0xFFF'

# because Venus simulator doesn't support large immediates well

# ===== Initialize 7-Segment Display Registers =====

# Set all displays to blank (0xFF = all segments off)

addi    s0, x0, 0xFF       # HEX0 = blank

addi    s1, x0, 0xFF       # HEX1 = blank

addi    s2, x0, 0xFF       # HEX2 = blank

addi    s3, x0, 0xFF       # HEX3 = blank

addi    s4, x0, 0xFF       # HEX4 = blank

addi    s5, x0, 0xFF       # HEX5 = blank

addi    s6, x0, 0xFF       # HEX6 = blank

addi    s7, x0, 0xFF       # HEX7 = blank

# =================== DECIMAL CONVERSION (HEX0-3) ===================

# Convert the 12-bit value to decimal and display on HEX0-3

addi    t2, t1, 0          # t2 = input value (n)

beq     t2, x0, dec_is_zero # If n == 0, handle special case

addi    t3, x0, 4          # t3 = 4 (maximum decimal digits to display)

addi    a3, x0, 0          # a3 = digit counter (0-3 for HEX0-3)

dec_loop:

beq     t2, x0, done_dec   # If n == 0, finished conversion

beq     a3, t3, done_dec   # If 4 digits processed, done

# ----- Integer Division by 10 (Using Repeated Subtraction) -----

# Calculate: quotient = n / 10, remainder = n % 10

addi    a0, t2, 0          # a0 = dividend (n)

addi    a1, x0, 0          # a1 = quotient = 0

addi    a2, x0, 10         # a2 = divisor = 10

div10_dec:

bltu    a0, a2, enddiv_dec # If dividend < 10, division complete

addi    a0, a0, -10        # dividend -= 10

addi    a1, a1, 1          # quotient += 1

jal     x0, div10_dec      # Loop back

enddiv_dec:

# Now: a0 = remainder (0-9), a1 = quotient

# ----- Map Digit (0-9) to 7-Segment Code -----

jal     ra, map_hex_digit  # Call mapping function, result in a0

# ----- Store 7-Segment Code in Appropriate Register (s0-s3) -----

addi    a4, x0, 0          # Check digit position

beq     a3, a4, st_dec0    # If position 0, store to s0 (HEX0)

addi    a4, x0, 1

beq     a3, a4, st_dec1    # If position 1, store to s1 (HEX1)

addi    a4, x0, 2

beq     a3, a4, st_dec2    # If position 2, store to s2 (HEX2)

addi    a4, x0, 3

beq     a3, a4, st_dec3    # If position 3, store to s3 (HEX3)

jal     x0, after_store_dec

st_dec0:

addi s0, a0, 0             # HEX0 = 7-segment code

jal x0, after_store_dec

st_dec1:

addi s1, a0, 0             # HEX1 = 7-segment code

jal x0, after_store_dec

st_dec2:

addi s2, a0, 0             # HEX2 = 7-segment code

jal x0, after_store_dec

st_dec3:

addi s3, a0, 0             # HEX3 = 7-segment code

jal x0,  after_store_dec

after_store_dec:

addi    a3, a3, 1          # Increment digit counter

addi    t2, a1, 0          # n = quotient (for next iteration)

jal     x0, dec_loop       # Continue loop

dec_is_zero:

# Special case: if input is 0, display '0' on HEX0

addi    s0, x0, 0xC0       # 7-segment code for '0'

addi    a3, x0, 1          # Set digit counter to 1

done_dec:

# =================== HEXADECIMAL CONVERSION (HEX4-7) ===================

# Display the 12-bit value as hexadecimal on HEX4-7

# ----- HEX4: Display lowest 4 bits (bits 3-0) -----

andi    a0, t1, 0xF        # Extract bits [3:0]

jal     ra, map_hex_digit  # Convert to 7-segment code

addi    s4, a0, 0          # Store to HEX4

# ----- HEX5: Display bits 7-4 -----

srli    t6, t1, 4          # Shift right 4 bits

andi    a0, t6, 0xF        # Extract bits [3:0] (originally bits [7:4])

jal     ra, map_hex_digit  # Convert to 7-segment code

addi    s5, a0, 0          # Store to HEX5

# ----- HEX6: Display bits 11-8 -----

srli    t6, t1, 8          # Shift right 8 bits

andi    a0, t6, 0xF        # Extract bits [3:0] (originally bits [11:8])

jal     ra, map_hex_digit  # Convert to 7-segment code

addi    s6, a0, 0          # Store to HEX6

# ----- HEX7: Display bits 15-12 -----

# (Should be 0 for 12-bit input, but kept for completeness)

srli    t6, t1, 12         # Shift right 12 bits

andi    a0, t6, 0xF        # Extract bits [3:0] (originally bits [15:12])

jal     ra, map_hex_digit  # Convert to 7-segment code

addi    s7, a0, 0          # Store to HEX7

# =================== PACK AND WRITE TO HARDWARE ===================

pack_and_write:

# ----- Pack HEX0-3 into 32-bit word (word_lo) -----

# Format: [HEX3][reserved][HEX2][reserved][HEX1][reserved][HEX0][reserved]

slli    t1, s3, 24         # HEX3 in bits [31:24]

slli    t2, s2, 16         # HEX2 in bits [23:16]

or      t1, t1, t2         # Combine HEX3 and HEX2

slli    t2, s1, 8          # HEX1 in bits [15:8]

or      t1, t1, t2         # Combine with HEX1

or      t1, t1, s0         # Combine with HEX0 in bits [7:0]

# ----- Pack HEX4-7 into 32-bit word (word_hi) -----

# Format: [HEX7][reserved][HEX6][reserved][HEX5][reserved][HEX4][reserved]

slli    t2, s7, 24         # HEX7 in bits [31:24]

slli    t3, s6, 16         # HEX6 in bits [23:16]

or      t2, t2, t3         # Combine HEX7 and HEX6

slli    t3, s5, 8          # HEX5 in bits [15:8]

or      t2, t2, t3         # Combine with HEX5

or      t2, t2, s4         # Combine with HEX4 in bits [7:0]

# ===== Check Individual Display Enable Bits =====

# ----- Check SW[14] (HEX display enable) -----

srli    t6, s8, 14         # Get bit SW[14]

andi    t6, t6, 1          # Mask to get only bit 14

bne     t6, x0, write_hex  # If SW[14] == 1, keep HEX4-7 enabled

li      t2, 0xFFFFFFFF     # If SW[14] == 0, blank HEX4-7 (all segments off)

write_hex:

# ----- Check SW[13] (DEC display enable) -----

srli    t6, s8, 13         # Get bit SW[13]

andi    t6, t6, 1          # Mask to get only bit 13

bne     t6, x0, write_dec  # If SW[13] == 1, keep HEX0-3 enabled

li      t1, 0xFFFFFFFF     # If SW[13] == 0, blank HEX0-3 (all segments off)

write_dec:

# ----- Write to 7-Segment Display Registers -----

sw      t1, 0(t4)          # Write word_lo to HEX_LO (HEX0-3)

sw      t2, 0(t5)          # Write word_hi to HEX_HI (HEX4-7)

jal     x0, main_loop      # Loop back to read switches again

# ===== Disabled State: Clear All 7-Segment Displays =====

disabled:

li      t1, 0xFFFFFFFF     # Load blank pattern (all segments off)

sw      t1, 0(t4)          # Clear HEX0-3

sw      t1, 0(t5)          # Clear HEX4-7

jal     x0, main_loop      # Loop back to main

# ==============================================================================================

# Subroutine: map_hex_digit

# ==============================================================================================

# Description: Maps a hexadecimal digit (0-F) to its 7-segment display code

# Input:  a0 = digit value (0-15)

# Output: a0 = 7-segment code (active-low, 0=on, 1=off)

# Uses:   t0 (temporary for comparison)

# ==============================================================================================

map_hex_digit:

# ----- Check digit value and jump to appropriate code -----

addi    t0, x0, 0

beq     a0, t0, md0        # If digit == 0

addi    t0, x0, 1

beq     a0, t0, md1        # If digit == 1

addi    t0, x0, 2

beq     a0, t0, md2        # If digit == 2

addi    t0, x0, 3

beq     a0, t0, md3        # If digit == 3

addi    t0, x0, 4

beq     a0, t0, md4        # If digit == 4

addi    t0, x0, 5

beq     a0, t0, md5        # If digit == 5

addi    t0, x0, 6

beq     a0, t0, md6        # If digit == 6

addi    t0, x0, 7

beq     a0, t0, md7        # If digit == 7

addi    t0, x0, 8

beq     a0, t0, md8        # If digit == 8

addi    t0, x0, 9

beq     a0, t0, md9        # If digit == 9

addi    t0, x0, 10

beq     a0, t0, mdA        # If digit == A

addi    t0, x0, 11

beq     a0, t0, mdB        # If digit == B

addi    t0, x0, 12

beq     a0, t0, mdC        # If digit == C

addi    t0, x0, 13

beq     a0, t0, mdD        # If digit == D

addi    t0, x0, 14

beq     a0, t0, mdE        # If digit == E

addi    t0, x0, 15

beq     a0, t0, mdF        # If digit == F

# Default case: invalid digit

addi    a0, x0, 0x00       # Return 0x00 (all segments on) for invalid input

jalr    x0, ra, 0          # Return

# ----- 7-Segment Codes (Active-Low: 0=segment on, 1=segment off) -----

# Segment layout:  a

#                 f b

#                  g

#                 e c

#                  d

# Bit order: [dp g f e d c b a] (bit 7 not used for decimal point)

md0:

addi a0, x0, 0xC0          # '0': segments a,b,c,d,e,f on

jalr x0, ra, 0

md1:

addi a0, x0, 0xF9          # '1': segments b,c on

jalr x0, ra, 0

md2:

addi a0, x0, 0xA4          # '2': segments a,b,d,e,g on

jalr x0, ra, 0

md3:

addi a0, x0, 0xB0          # '3': segments a,b,c,d,g on

jalr x0, ra, 0

md4:

addi a0, x0, 0x99          # '4': segments b,c,f,g on

jalr x0, ra, 0

md5:

addi a0, x0, 0x92          # '5': segments a,c,d,f,g on

jalr x0, ra, 0

md6:

addi a0, x0, 0x82          # '6': segments a,c,d,e,f,g on

jalr x0, ra, 0

md7:

addi a0, x0, 0xF8          # '7': segments a,b,c on

jalr x0, ra, 0

md8:

addi a0, x0, 0x80          # '8': all segments on

jalr x0, ra, 0

md9:

addi a0, x0, 0x90          # '9': segments a,b,c,d,f,g on

jalr x0, ra, 0

mdA:

addi a0, x0, 0x88          # 'A': segments a,b,c,e,f,g on

jalr x0, ra, 0

mdB:

addi a0, x0, 0x83          # 'b': segments c,d,e,f,g on

jalr x0, ra, 0

mdC:

addi a0, x0, 0xC6          # 'C': segments a,d,e,f on

jalr x0, ra, 0

mdD:

addi a0, x0, 0xA1          # 'd': segments b,c,d,e,g on

jalr x0, ra, 0

mdE:

addi a0, x0, 0x86          # 'E': segments a,d,e,f,g on

jalr x0, ra, 0

mdF:

addi a0, x0, 0x8E          # 'F': segments a,e,f,g on

jalr x0, ra, 0